DIGITAL LOGIC & COMPUTER ORGANIZATION
DETAILED ANSWERS

Based on the questions provided in CO.docx

Note: Repeated questions in the original question paper are answered once in detail and cross-referenced where appropriate.

# 1. Number Systems and Conversions

A number system is a method of representing numbers using a fixed set of symbols and a base (radix). The commonly used number systems in digital electronics are:

Decimal (base 10): digits 0–9.

Binary (base 2): digits 0 and 1; used internally by digital computers.

Octal (base 8): digits 0–7; each octal digit corresponds to 3 binary bits.

Hexadecimal (base 16): digits 0–9 and A–F; each hexadecimal digit corresponds to 4 binary bits.

For an integer N in base r, its decimal value is obtained by multiplying each digit by the corresponding power of r.

(dₙ…d₁d₀)ᵣ = dₙrⁿ + … + d₁r¹ + d₀r⁰

a) Convert (745)₈

(745)₈ = 7×8² + 4×8¹ + 5×8⁰ = 448 + 32 + 5 = (485)₁₀

Octal → Binary: 7=111, 4=100, 5=101  ⇒  (745)₈ = (111100101)₂

Binary → Hexadecimal: group from the right: 111 100 101 → 0001 1110 0101 = 1E5.

(745)₈ = (111100101)₂ = (485)₁₀ = (1E5)₁₆

b) Convert (3AF)₁₆

(3AF)₁₆ = 3×16² + 10×16 + 15 = 768 + 160 + 15 = (943)₁₀

Hex → Binary: 3=0011, A=1010, F=1111  ⇒  (001110101111)₂ = (1110101111)₂

Binary → Octal: group into 3 bits: 001 110 101 111 → 1 6 5 7.

(3AF)₁₆ = (1110101111)₂ = (943)₁₀ = (1657)₈

c) Convert (11010110)₂

(11010110)₂ = 1×2⁷+1×2⁶+0×2⁵+1×2⁴+0×2³+1×2²+1×2¹+0×2⁰ = 214

Binary → Octal: 011 010 110 → 326.

Binary → Hexadecimal: 1101 0110 → D6.

(11010110)₂ = (326)₈ = (214)₁₀ = (D6)₁₆

# 2. Express F = A + B̄C as a Sum of Minterms (SOP)

Given: F = A + B̄C. Assume the variable order is A, B, C.

Construct the truth table and identify all input combinations for which F=1.

| A | B | C | B̄C | F |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 0 | 1 |

The 1s occur at minterms m₁, m₄, m₅, m₆ and m₇.

F(A,B,C) = Σm(1,4,5,6,7)

Canonical SOP:

F = ĀB̄C + AB̄C̄ + AB̄C + ABC̄ + ABC

# 3. Boolean Algebra Simplification: XY + X(Y+Z) + Y(Y+Z)

X·Y + X(Y+Z) + Y(Y+Z)

= XY + (XY + XZ) + (Y + YZ)

Using the distributive law and absorption law:

= XY + XY + XZ + Y + YZ

= XY + XZ + Y + YZ

Y + XY = Y

Y + YZ = Y

∴ F = Y + XZ

Therefore, the simplified expression is:

F = Y + XZ

# 4. Digital Logic Gates

Logic gates are basic building blocks of digital circuits. They operate on binary inputs (0 or 1) and produce a binary output.

The standard symbols can be drawn as follows in an exam: AND—D-shaped gate; OR—curved-input gate; NOT—triangle with an output bubble; NAND—AND with an output bubble; NOR—OR with an output bubble; XOR—OR with an additional curved line at the input; XNOR—XOR with an output bubble.

Boolean expressions and truth tables:

| A | B | AND AB | OR A+B | NAND (AB)̄ | NOR (A+B)̄ | XOR A⊕B | XNOR (A⊕B)̄ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 0 | 1 |
| 0 | 1 | 0 | 1 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 | 1 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 |

NOT gate:

| A | Y=Ā |
| --- | --- |
| 0 | 1 |
| 1 | 0 |

AND: Y = AB. Output is 1 only when all inputs are 1.

OR: Y = A + B. Output is 1 when at least one input is 1.

NOT: Y = Ā. Output is the complement of the input.

NAND: Y = (AB)̄. It is the complement of AND.

NOR: Y = (A+B)̄. It is the complement of OR.

XOR: Y = A⊕B = ĀB + AB̄. Output is 1 when inputs are different.

XNOR: Y = (A⊕B)̄ = AB + ĀB̄. Output is 1 when inputs are equal.

# 5 & 12. XOR and XNOR Using Only NAND Gates

A NAND gate is a universal gate, so it can be used to implement any Boolean function.

XOR implementation using four NAND gates:

N1 = A NAND B = (AB)̄

N2 = A NAND N1

N3 = B NAND N1

Y = N2 NAND N3

Y = A⊕B

Thus, the four-NAND realization produces XOR.

XNOR can be obtained by NAND-inverting the XOR output:

First generate X = A⊕B using the four NAND gates above.

Connect X to both inputs of a fifth NAND gate.

Y = X NAND X = X̄ = (A⊕B)̄ = A XNOR B.

XNOR = (A⊕B)̄

Hence XOR requires four NAND gates in the standard construction, and XNOR requires one additional NAND gate.

# 6 & 18. Basic Structure of a Computer

A computer is an electronic system that accepts data and instructions, processes them according to the instructions, stores information, and produces results.

The basic structure consists of the following major units:

Input Unit: accepts data and instructions from devices such as keyboard, mouse, scanner, sensors, etc., and converts them into a form suitable for processing.

Central Processing Unit (CPU): performs processing and controls overall operation. It consists mainly of the Arithmetic and Logic Unit (ALU), Control Unit (CU), and registers.

Memory/Storage Unit: stores programs, input data, intermediate results, and final results. Primary memory includes RAM and ROM; secondary storage includes SSDs, HDDs and other storage devices.

Output Unit: converts processed information into human-readable or usable form through monitors, printers, speakers, etc.

System Interconnection: buses/interconnects provide communication paths among CPU, memory and I/O units.

A simple block representation is:

Input Unit → CPU ↔ Memory Unit → Output Unit

CPU = Control Unit + ALU + Registers

# 7. Conversion of Decimal Fractions into Binary Fractions

To convert a decimal fraction into a binary fraction, repeatedly multiply the fractional part by 2. At each step, record the integer part (0 or 1). Continue until the fractional part becomes zero or the required number of binary places is obtained.

Multiply the decimal fraction by 2.

Record the integer part as the next binary digit.

Discard the integer part and use the remaining fractional part for the next multiplication.

Repeat the process.

Read the recorded binary digits from top to bottom.

Example: Convert (0.625)₁₀ to binary.

| Step | Fraction × 2 | Integer bit | New fraction |
| --- | --- | --- | --- |
| 1 | 0.625×2=1.250 | 1 | 0.250 |
| 2 | 0.250×2=0.500 | 0 | 0.500 |
| 3 | 0.500×2=1.000 | 1 | 0.000 |

(0.625)₁₀ = (0.101)₂

Example with a non-terminating result: 0.3₁₀ gives 0.0100110011…₂, so a suitable number of bits is retained depending on required precision.

# 8 & 15. Boolean Algebra Simplification

F = ĀBC + AB̄C + ABC̄ + ABC

The four terms correspond to minterms m₃, m₅, m₆ and m₇. This is the odd/even pattern of a 3-variable majority function with output 1 for inputs having at least two 1s.

Group the terms:

F = ABC̄ + ABC + ĀBC + AB̄C

= AB(C̄+C) + BC(Ā+A) + AB̄C

= AB + BC + AB̄C

Using X + X̄Y = X + Y:

AB + AB̄C = A(B + B̄C) = A(B+C)

= AB + AC

Therefore, F = AB + AC + BC

Final simplified expression:

F = AB + AC + BC

# 9. De Morgan’s Theorems

De Morgan’s theorems are fundamental Boolean identities used to complement AND/OR expressions.

First theorem:

(A + B)̄ = ĀB̄

Second theorem:

(AB)̄ = Ā + B̄

Verification by truth table:

| A | B | A+B | (A+B)̄ | Ā | B̄ | ĀB̄ | AB | (AB)̄ | Ā+B̄ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 1 | 1 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |

The columns (A+B)̄ and ĀB̄ are identical, proving the first theorem. The columns (AB)̄ and Ā+B̄ are identical, proving the second theorem.

Boolean-algebra verification:

(A+B)(Ā+B̄) = AB̄ + ĀB

More directly, using complement and distributive laws, the identities satisfy the defining complement conditions. De Morgan’s theorems can also be generalized to any number of variables.

# 10. SOP vs POS Simplification Using Karnaugh Maps

A Karnaugh map (K-map) is a graphical method for simplifying Boolean expressions by grouping adjacent cells.

| Feature | SOP | POS |
| --- | --- | --- |
| Meaning | Sum of Products | Product of Sums |
| K-map cells grouped | 1s | 0s |
| Each group produces | Product term | Sum term |
| Final operation | OR of product terms | AND of sum terms |
| Canonical form | Σm (sum of minterms) | ΠM (product of maxterms) |

SOP simplification procedure:

Place 1s in the K-map for the minterms where F=1.

Group adjacent 1s in powers of two: 1, 2, 4, 8, etc.

Make groups as large as possible and allow overlapping when useful.

For each group, retain variables that remain constant.

OR all resulting product terms.

POS simplification procedure:

Place 0s in the K-map for the maxterms where F=0.

Group adjacent 0s in powers of two.

For each group, retain variables that remain constant.

Form the corresponding sum term; a variable that is 0 in the group appears uncomplemented in the sum, while a variable that is 1 appears complemented.

AND all resulting sum terms.

Thus, SOP focuses on grouping 1s and produces an OR of AND terms, whereas POS focuses on grouping 0s and produces an AND of OR terms.

# 11. Basic Structure and Functional Components of a Computer System

The functional organization of a computer system can be divided into input, processing, storage and output functions.

Input subsystem: supplies data and instructions to the computer.

Processing subsystem: CPU executes instructions and performs arithmetic, logical and control operations.

Memory subsystem: holds instructions, data and intermediate/final results.

Output subsystem: presents the processed information.

Interconnection/bus system: transfers data, addresses and control signals between components.

CPU components:

ALU: performs arithmetic operations such as addition/subtraction and logical operations such as AND, OR, comparison and shifting.

Control Unit: fetches and decodes instructions and generates control signals for their execution.

Registers: small, high-speed storage locations inside the CPU used for operands, addresses, instructions and intermediate results.

The instruction-processing cycle is commonly described as fetch → decode → execute, with memory and I/O interactions as required.

# 13. Number System Conversions

a) Convert (245)₈

(245)₈ = 2×8² + 4×8 + 5 = 128 + 32 + 5 = (165)₁₀

(245)₈ = (010 100 101)₂ = (10100101)₂

(10100101)₂ = (245)₈ = (165)₁₀ = (A5)₁₆

b) Convert (3AF)₁₆

(3AF)₁₆ = (1110101111)₂ = (1657)₈ = (943)₁₀

c) Convert (10110)₂

(10110)₂ = 1×16 + 4 + 2 = (22)₁₀

(10110)₂ = (026)₈ = (16)₁₆ = (22)₁₀

# 14. F = A + B̄C in Sum of Minterms

This is the same Boolean-function question as Question 2.

F(A,B,C) = Σm(1,4,5,6,7)

F = ĀB̄C + AB̄C̄ + AB̄C + ABC̄ + ABC

# 16. Digital Logic Gates

This is the same logic-gates question as Question 4. The required gates are AND, OR, NOT, NAND, NOR, XOR and XNOR, with their expressions and truth tables given in Question 4 above.

Key expressions:

AND: Y=AB

OR: Y=A+B

NOT: Y=Ā

NAND: Y=(AB)̄

NOR: Y=(A+B)̄

XOR: Y=ĀB+AB̄

XNOR: Y=AB+ĀB̄

# 17. XOR and XNOR Using Only NOR Gates

NOR is also a universal gate. Therefore XOR and XNOR can be constructed using only NOR gates.

Let ↓ denote NOR.

XNOR using five NOR gates:

N1 = A ↓ B = (A+B)̄

N2 = A ↓ N1

N3 = B ↓ N1

N4 = N2 ↓ N3 = A⊕B

N5 = N4 ↓ N4 = (A⊕B)̄ = A⊙B (XNOR)

N4 = XOR,   N5 = XNOR

Alternatively, if the desired output is XOR, stop at N4. If XNOR is required, invert the XOR output using one additional NOR gate whose two inputs are tied together.

Important NOR identities:

Ā = A NOR A

A+B = (A NOR B) NOR (A NOR B)

AB = (A NOR A) NOR (B NOR B)

# Quick Revision Summary

Number systems: Binary=2, Octal=8, Decimal=10, Hexadecimal=16.

F=A+B̄C = Σm(1,4,5,6,7).

XY+X(Y+Z)+Y(Y+Z) simplifies to Y+XZ.

ĀBC+AB̄C+ABC̄+ABC simplifies to AB+AC+BC.

De Morgan: (A+B)̄=ĀB̄ and (AB)̄=Ā+B̄.

SOP groups 1s in K-map; POS groups 0s.

NAND and NOR are universal gates.

Decimal fractions are converted to binary by repeated multiplication by 2.

— End of Detailed Answers —